CJAP — Runtime & Operating Cycle

The robot's turn cycle with its timeout, barge-in and error path at each step — and the untethered-runtime gap.

### Operating cycle

| Step | Enters when | Timeout / barge-in / error path | Duration |
| --- | --- | --- | --- |
| Idle | Boot, or previous turn ended | Alive gesture every 10 s. No timeout. Service auto-starts at boot | Measured — indefinite |
| Wake detected | openWakeWord score ≥ CJ_WAKE_OWW_THRESHOLD=0.08 in a 2.5 s window | Cooldown CJ_WAKE_COOLDOWN_S=0.1. Miss → operator force-listen from /maintain | Measured 0.33–0.6 s to “SPEAK NOW” |
| Listening | Immediately after wake | Closes after CJ_MIC_TRAILING_SILENCE_S=1.5 of silence. Captures with < CJ_VAD_MIN_SPEECH_S=0.4 of speech are discarded | Measured 1.6–3.1 s of real speech |
| Transcribing | Mic closes; sub-second “Ah.” acknowledgment plays | No automatic STT failover. Network loss → ~/fillers_bail/not_connected.wav | Measured 1.2–1.6 s |
| Routing | Transcript passes the gate | Truncated router JSON → anchor-routing fallback. Canned match skips this entirely | [TO CONFIRM: router seconds on robot ¦ owner: Pao] |
| Generating | Topics resolved; filler already playing | Filler holds up to CJ_FILLER_FIRST_WAIT_S=1.2. Credits exhausted → in-voice degraded line, no real answer | [TO CONFIRM: composer TTFT on robot ¦ owner: Pao] |
| Speaking | First sentence composed | Barge-in: wake phrase at CJ_STOP_OWW_THRESHOLD=0.01 cuts playback and returns to sleep. TTS failure → OpenAI TTS fallback | Measured first audio 4.9–6.2 s after transcript; first-sentence synth 1.7–2.5 s |
| Return to idle | Answer completes or is stopped | CJ_FOLLOWUP_WINDOW_S=0 — follow-up window disabled; next question needs a fresh wake | Measured — immediate |

End-to-end, measured: 4.9–6.2 s from transcript to first spoken audio (2026-08-21, after the latency-v3 changes; was 15–20 s before streaming). Target in README.md is ≤4 s end-to-end — not yet met on the robot.

### Untethered runtime

[TO CONFIRM: untethered runtime never measured — requires a timed discharge test ¦ owner: hardware] No battery capacity, runtime-on-full-charge, recharge time, or idle-vs-active power figure exists anywhere in the repository. A repository-wide search for battery, charge, discharge, mAh, watt and power-draw terms returned only corpus prose — no measurement, no specification, no test record. These figures are deliberately left blank rather than inferred from component datasheets: a datasheet figure would not account for the mic array, the wake model running continuously, Bluetooth audio, or the motors. Closing this needs one timed discharge test — full charge, run the normal turn cycle to shutdown, log wall-clock runtime and turn count, then a timed recharge.

Sources: CJAP repo pi/deployment-snapshots @ 2545882 — app/.env.example, deploy/pi/systemd/supervaise.service.d/wakeword.conf, deploy/pi/PROJECT_NOTES.txt, app/usage_meter.py, corpus/MANIFEST.md

Status: **Confirmed:** every threshold, timeout and env var above is verbatim from `wakeword.conf`; the durations marked Measured are from `PROJECT_NOTES.txt`. **Open:** router and composer timings on the robot; the entire untethered-runtime section.